# Solid Waste Management – Units 1, 2 & 3 Notes

## UNIT I: Introduction & Characteristics

Definition: Solid waste refers to unwanted discarded solid materials from residential, commercial, industrial and other activities.

Types: Municipal, industrial, hazardous, biomedical, agricultural, e-waste and construction waste.

Physical Characteristics: Density, moisture content, particle size, calorific value and porosity affect handling and treatment.

Chemical Characteristics: Ultimate analysis (C, H, O, N, S) and proximate analysis (moisture, volatile matter, ash, fixed carbon), pH and toxic elements.

Biological Characteristics: Biodegradability, microbial activity and compostability.

Effects on Health & Environment: Spread of diseases, air and water pollution, land degradation and greenhouse gas emissions.

Variation of Composition: Depends on income level, lifestyle, season, location and population density.

Material Flow in Society: Raw materials → production → consumption → waste → recycling loop.

Reduction in Raw Material & Waste Generation: Source reduction, reuse, recycling and efficient resource utilization.

Reuse & Material Recovery: Recycling of paper, plastic, metals; composting and energy recovery.

## UNIT II: Engineered Systems

Typical Generation Rates: 0.3–0.8 kg/person/day in Indian cities; varies with income and urbanization.

Estimation Methods: Load count, weight-volume analysis and per capita method.

Factors Affecting Generation: Population, economic status, lifestyle, climate, industrial growth and festivals.

On-site Handling: Segregation, reuse, recycling and storage at source.

Storage of Waste: Household bins, community containers; must be covered, durable and leak-proof.

Collection Systems: Door-to-door, community bin and block collection using hand carts and trucks.

Transfer & Transport: Transfer stations used to shift waste from small to large vehicles for efficient transport.

## UNIT III: Processing Techniques

Mechanical Volume Reduction: Compaction, baling, shredding and grinding to reduce waste volume and improve handling.

Thermal Volume Reduction: Includes incineration and pyrolysis.

Incineration: Controlled combustion of waste with excess oxygen at high temperature; reduces volume and produces energy.

Pyrolysis: Heating waste in absence of oxygen to produce gas, oil and char with lower pollution than incineration.